Additive Manufacturing 2026

SkillsUSA State Challenge

Have questions: mdangelo@sme.org  Coming soon website link team resources.

Welcome to the “Mini-Figure Catapult/Trebuchet Challenge” challenge!

The Additive Manufacturing (AM) contest evaluates students’ ability to design, prototype, validate, and communicate a functional mechanical device using only 3D-printed components.

Competitors must design a catapult or trebuchet capable of launching a standard LEGO® minifigure using no added materials, relying exclusively on the properties of 3D-printed plastics

Competitors create a fully additively manufactured launcher—catapult or trebuchet—where all energy storage and mechanical action originate from 3D-printed geometry alone, including:

- Printed torsion beams

- Printed elastic flexures

- Printed springs or spring arms

- Printed counterweights (printed material only)

- Printed tension/flexural systems

No external rubber bands, cords, metals, or weights may be used.

Competition Requirements

Design Requirements

Each team must design a launcher that:

- Launches a standard LEGO® minifigure (supplied by contest judges).

- Stores and releases energy using only 3D-printed material behavior:

  - Elastic bending

  - Torsional flexure

  - Printed leaf or coil springs

  - Beam deflection

  - Printed counterweight (printed plastic only)

  - No metal, rubber, glue, magnets, or external energy inputs.

- Operates using a catapult or trebuchet mechanism:

  - Catapult: energy stored in flexural or torsional arms.

  - Trebuchet: energy stored in printed counterweight or pendulum motion.

- Has a locking trigger mechanism

- Is safe and stable during operation.

- Requires no fasteners—all connections must be printed or snap-fit.

- May be a single print or multi-component assembly (can use multiple 3D printed material):

  - Bonus points for functionally integrated, single-print solutions.

Size & Print Constraints

- Fully assembled launcher must fit inside: 150 × 150 × 150 mm

- All components must be 100% 3D-printed

- Any 3D printer technology may be used:

  - FDM, SLA, DLP, PolyJet, SLS, SAF, MJF, etc.

- Any printable material is allowed:

  - PLA, ABS, TPU, Nylon, resin, PA11/12, etc.

- If multiple materials are used, all still must be 100% AM-produced.

Performance Requirements

During testing, each launcher must:

- Securely hold and release the minifigure.

- Launch the figure forward, not upward only.

- Complete three launches, which are measured for:

  - Distance

  - Repeatability (all three launches must land within a 6-inch diameter of first launch.

  - Safety and stability (No part interference, damage during operation, or unsafe trajectories)

Design Considerations

4.1 Energy Storage Methods (Allowed)

Teams may use:

- Printed flexible beams (like bow limbs)

- Printed torsion rods

- Printed leaf springs

- Printed coil springs

- Printed counterweights (solid or hollow filled only with printed material)

- Printed whip-action arms

4.2 Energy Storage Methods (NOT Allowed)

- Rubber bands

- Metal or lead weights

- Ball bearings

- Strings or ropes

- Electronics

- External kinetic input beyond human-triggered release

4.3 Stability & Safety

- Broad base recommended

- Ensure center of mass remains within footprint

- Avoid high-tension prints that may crack explosively

- Ensure minifigure is not struck by structural components during firing

4.4 AM Material Behavior Tips

- FDM: stronger along filament path; avoid layer-splitting by orienting beams horizontally

- Resin: brittle; great for fine details but avoid ultra-high deflection

- SAF/SLS: excellent for small flexures; nylon is durable

- Multi-material: optimize stiffness vs elasticity

4.5 Recommended Geometries

- Arched, tapered arms for elastic bending

- Hollow counterweights (printed-only mass)

- Integrated bearings or bushing pockets

- Snap-fit axles and pivot joints

- Lattice structures for energy absorption

State Competition Procedure

Before or on contest day:

- Students submit Engineering Notebook (Engineering notebook guidelines below)

- Students submit print files in both CAD (.step, .iges, .sldprt, etc.) and mesh (STL, 3MF, OBJ, etc.) format to Contests@SkillsUSAnc.org

- Students submit physical parts

- Students submit final assembly if applicable

- Students submit their Presentation

State Competition Judging Criteria

- The Engineering Notebook should contain robust content, including at a minimum the following:

  - Be clearly labeled with contestant name(s), date and page # on each page

  - Begin with a problem statement

  - Include discovery and documentation of approach to solve problem

  - Include sketched design concepts with critical features labeled

  - Critical dimensions clearly labeled in design sketch

  - Considerations for designing for additive manufacturing distinctly addressed (i.e. part strength, part orientation) especially including any expected risks during printing

  - Screenshots of the print time and material usage for all printed parts

  - Design decisions and alternatives are documented and evaluated thoughtfully

- The design must adhere to the Competition Requirements stated in the prior page.

- Quality of final assembly

  - Does it perform the function in the manner it was designed to do?

  - Does it meet all requirements in contest guidelines?

  - Do inserted components or multiple printed parts mates together properly?

  - Did the students design the part with additive manufacturing in mind?

  - Is there sufficient tolerance between parts for movement?

- The design must illustrate best practices for “design for additive manufacturing (DFAM)”. Below are some potential DFAM metrics to optimize for.

  - Build Time

  - Post-Processing/Support Removal Time

  - Functionality Optimization (gear ratio, pliability, strength, etc.)

  - Monetary Savings

  - Material Consumption

  - Energy Usage

  - Component Consolidation (lack of store-bought hardware)

  - Lightweighting for Ergonomics

- Presentation Criteria

  - The team clearly describes their understanding of the problem to be solved.

  - Design Process: good design logic is used for key design choices. Intentional and well-communicated

  - The presentation is professional and well-rehearsed

  - The presentation emphasizes quantitative improvements (measured and estimated) of the time, quality, or cost of the improvement as well as any DFAM tactics employed.

  - Practical evaluation: team demonstrates visually (videos, photos, drawings, animation, etc.) the task they improved, both before and after.

Grading Rubric

GR.1 Launch Performance — 35 pts

- Distance (best of 3 launches): 25 pts

  - Compare to peer competitors

- Consistency: 5 pts (all 3 figures land within a 6-inch diameter)

- Safe trajectory & controlled release (no interference or damage to parts while operating): 5 pts

GR.2 DFAM Execution & Print Quality — 20 pts

- Efficient use of AM geometry

- Thoughtful layer orientation

- Integrated mechanisms (springs, hinges, etc.)

- Complexity that adds functional value

- Built in locking trigger

- Clean print

- Quick assembly

- Good tolerance

GR.3 Engineering Notebook — 15 pts

- Completeness

- Technical clarity

- Analysis & iteration

- CAD quality

GR.4 Presentation — 15 pts

GR.5 Knowledge Exam — 15 pts

Pass with 70% or better or Fail

Additive Manufacturing Technology Resources:

The national technical committee provides a webpage with resources available for the previous NLSC.

State technical chairs, advisors, and students can use these resources to prepare for the competition. For more information visit: https://skillsusa-amchallenge.my.canva.site/